COMMISSIONING AND OPERATIONS MANUAL

Installation, Calibration,
and Operation Guide

Delta X Software - Multi-Robot Sorting on a Shared Conveyor

Applies to Delta X Software 1.3.0 and the multi-robot tracking platform

Audience: system integrators, commissioning engineers, and lead operators

# Document Control

| Field | Value |
| --- | --- |
| Document | Delta X Multi-Robot Installation, Calibration, and Operation Guide |
| Revision | 0.2 - Release candidate |
| Date | 2026-08-24 |
| Scope | One camera, one encoder, one conveyor, and multiple delta robots using shared tracking |
| Example assets | script-example/multi-robot-sorting/ |

SAFETY: G-Script, vision, and tracking are not safety functions. E-stop, guards, axis limits, end-effector energy isolation, and collision prevention must be implemented by an appropriate safety circuit, PLC, or controller.

## How to Use This Guide

- System integrators should follow Sections 1 through 9 during initial commissioning.

- Operators should use Sections 10 through 12 and the acceptance checklist in Section 13.

- Every calibration section has an acceptance gate. Do not proceed until the previous gate passes.

- Numeric values in sample programs are starting points and must be replaced with measurements from the actual cell.

## Contents

- System architecture and coordinate conventions

- Hardware installation

- Software installation

- Project and device setup

- Encoder calibration

- Camera and image-region calibration

- Camera P to conveyor C mapping

- Conveyor C to robot Ri mapping

- Detector and tracking configuration

- Multi-robot G-Script configuration

- Operation, stopping, and fault recovery

- Acceptance and maintenance

# 1. System Architecture and Coordinate Conventions

The system uses one shared moving-object map. The camera produces detections; encoder-aware tracking converts detections into stable UIDs; each robot is an independent worker that obtains ownership through an atomic claim.

Figure 1 - Camera to detector to tracking to shared object map to robot workers

## Keep three coordinate systems separate

| System | Unit | Convention and use |
| --- | --- | --- |
| P | pixel | Camera coordinates after resize, crop, and warp; used only before mapping. |
| C | mm | Shared conveyor system; use +Y along belt travel and +X across the belt. |
| Ri | mm/deg | Robot i local coordinates; every robot G01 move executes in this system. |

RULE: Define claim zones in C. Only after a successful claim may a worker map the pick point from C to Ri with ConveyorToRobotN.Map(x,y).

Figure 2 - UID lifecycle and owner/lease protection against duplicate picks

# 2. Hardware Installation

## 2.1 Recommended mechanical layout

- Mount the encoder on a non-slip shaft or a measuring wheel with stable pressure. Do not use motor pulses when the transmission may slip relative to the belt.

- Place the camera far enough upstream for inference, communication, and robot approach. Rigidly mount camera, lens, and lighting as one assembly.

- Install robots in conveyor order. Keep robot workspaces non-overlapping until a collision coordinator is available.

- Separate encoder and camera cables from power cables; terminate shielding according to the manufacturer requirements.

- Verify that E-stop, guards, mechanical limits, safe torque off, and vacuum release operate independently of the PC.

## 2.2 Naming convention

| Device | Software name | Related variable or matrix |
| --- | --- | --- |
| Camera/detect | detect0 | tracking0, #Objects |
| Encoder | encoder0 | tracking0.VelocityVector |
| Conveyor | conveyor0 | +Y of C |
| First robot | robot0 | #0.ConveyorToRobot0, #R0... |
| Second robot | robot1 | #0.ConveyorToRobot1, #R1... |

ACCEPTANCE: Moving the belt by hand in the production direction increases C.Y. If it does not, reverse the sign in exactly one place.

# 3. Installing Delta X Software

## 3.1 Windows operator station

- Windows 10/11 64-bit and Microsoft Visual C++ Redistributable 2015-2022 x64.

- A portable package includes the Qt runtime, OpenCV DLL, GScript_Documentation.html, script-example/, and plugin/. Models and proprietary camera runtimes are installed separately.

- Install Basler pylon Runtime or Hikrobot MVS Runtime before using the corresponding GigE/USB3 Vision camera.

- Install 64-bit Python when using an external detector or YOLO; use a dedicated virtual environment for the cell.

py -3.10 -m venv %LOCALAPPDATA%\DeltaX\py
%LOCALAPPDATA%\DeltaX\py\Scripts\pip install ^
  ultralytics==8.0.200 opencv-python==4.9.0.80 numpy==1.26.4

## 3.2 Build from source (development station)

- Visual Studio 2022 MSVC v143, workload Desktop development with C++.

- Qt 6 for MSVC 2022 x64 with Serial Port, Multimedia, Svg, and Svg Widgets.

- Rerun qmake after changing a Qt kit to avoid stale generated ui_*.h files.

- Build Release, run windeployqt, and then use the repository packaging script to include verified runtime assets.

Figure 3 - Settings > Device: default COM port, baud rate, timeout, and reconnect

DO NOT SAVE YET: Do not click Save Settings until COM ports, baud rates, timeout, and the default camera have been verified. Back up settings before a major change.

# 4. Project and Device Setup

- Open Delta X Software, click + to create a project, and save it with an unambiguous cell or line name.

- On Robot, add robot0, robot1, and so on; select the controller COM port, baud rate, model, and degrees of freedom.

- On Conveyor, add conveyor0 and select direct G-code control or robot output according to the wiring design.

- On Encoder, add encoder0, select the X/Y axis, link conveyor0 when required, and enable Active.

- In Object Detector, create detect0, select Webcam / Industrial Camera / Images / Socket, and attach tracking0.

Figure 4 - Robot: connection, homing, pose, jogging, and motion parameters

JOGGING: For first motion, use a safe Z, 0.1-1 mm steps, and low speed. Verify X/Y/Z/W signs before homing or running a program.

Figure 5 - Conveyor: Manual Control mode, speed, distance, and target position

Figure 6 - Encoder: conveyor link, timer, velocity, position, reset, and scheduled G-code

# 5. Encoder Calibration

The encoder is the authoritative source for conveyor travel. Do not replace it with PC-time interpolation: the conveyor accelerates, decelerates, and may slip.

## 5.1 Determine mm/count and sign

- Mark two belt positions 500-1000 mm apart; stop at the first position and reset the encoder.

- Move in the production direction to the second position and record the count or reported-position difference.

- Calculate mm_per_count = measured distance / count difference. Example: 1000 mm / 20000 counts = 0.05 mm/count.

- Repeat forward and reverse travel at least five times. If error changes with load, correct mechanical slip before software tuning.

- Verify sign: travel along +Y must increase C.Y. Reverse the sign in the encoder configuration or Reverse Value, never in several locations.

| Test | Measured value | Requirement |
| --- | --- | --- |
| 1000 mm travel | ________ mm | Error below half of the pick tolerance |
| Five repetitions | Max-Min: ________ | No cumulative error growth |
| Direction reversal | Difference: ________ | Backlash is within budget |
| Encoder loss | Detection time: ________ | No new claims are issued |

ACCEPTANCE: Cumulative camera-to-pick error is below the end-effector error budget, and workers receive no new object when encoder data is stale.

# 6. Camera and Image-Region Calibration

Fix resolution, focus, exposure, ROI, and camera position before collecting points. Any resize, crop, or warp change after calibration invalidates the matrix.

Figure 7 - Object Detector image view, camera tools, and Calibration panel

## 6.1 Correct perspective with a chessboard

- Place a flat, undamaged chessboard on the belt plane and cover the camera work area.

- Click Find Chessboard. Select Edit chessboard for an existing setup or Find new chessboard for a new one.

- Enter the correct row/column count and square size; verify every detected corner.

- Correct misplaced corners with the adjustment tool, then enable Transform Image.

- Enable Crop Area and retain only the object path ROI; lock width, height, and exposure.

Figure 8 - Find Chessboard: create or edit configuration

Figure 9 - Select the measured chessboard dimensions

Figure 10 - Enable Transform Image after all corners are correct

Figure 11 - Cropped ROI containing only the usable conveyor region

ACCEPTANCE: Straight belt references remain straight throughout the ROI, perspective is negligible, and parts are not blurred at production speed.

# 7. Map Camera P to Conveyor C

P-to-C mapping converts a detection centre from pixels to millimetres. The two-point model is appropriate after perspective correction leaves only rotation, scale, and translation.

Figure 12 - Calibration: P1/P2, real coordinates, offset, distance, and matrix calculation

## 7.1 Two-point procedure

- Enable Select Calibration Points, choose a clear belt reference T, and copy its image coordinates into P1.

- Enter the measured P1' coordinates in C. If the encoder was reset at T, use X=0 and Y=0.

- Move the conveyor exactly 80-100 mm using a measured reference or encoder travel.

- Select the same reference T again and copy its image coordinates into P2; enter P2' with unchanged X and encoder travel as Y.

- Click Calculate Mapping Matrix, assign a stable name, and use Test Calibration Point on points excluded from the fit.

Figure 13 - Select the Calibration Point tool

Figure 14 - Select reference T for P1

Figure 15 - Move the belt through a measured 100 mm travel

Figure 16 - Select the same reference T for P2

UPGRADE PATH: If ROI-edge error is worse than centre error, use Cloud Point Mapping across the full ROI. Do not hide calibration error by increasing Distance Threshold.

# 8. Map Conveyor C to Each Robot Ri

Every robot requires a separate matrix because position, mounting angle, and origin differ. Store matrices in the tracking group, for example #0.ConveyorToRobot0 and #0.ConveyorToRobot1.

Figure 17 - Point Tool: Tracking Manager, velocity vector, and source/destination matrix

- Choose two widely separated belt references away from the edge of the robot workspace.

- At each reference, record C coordinates as Source; jog the robot to the same reference and record Ri as Destination.

- Click Calculate/Add Matrix and save it as the matching #0.ConveyorToRobotN.

- Validate at least three points excluded from fitting, including two pick-zone corners and one centre point.

- Teach SafeZ, PickZ, PlaceX/Y/Z, AngleOffset, and angle sign independently for each robot.

- Transform belt velocity from C to Ri. The SYNC vector is mm/s in robot axes, never counts/s or pixels/s.

Figure 18 - Encoder and velocity vector in Tracking Manager

| Robot i parameter | Taught value | Verification |
| --- | --- | --- |
| ConveyorToRobotN | ________________ | Maximum error ______ mm |
| SafeZ / PickZ | ______ / ______ mm | No collision / reliable pickup |
| Place XYZ | ________________ | Correct bin and recipe |
| AngleOffset | ______ deg | Correct at 0/45/90 deg |
| BeltVector | (____, ____, ____) mm/s | Robot follows the correct direction |

# 9. Detector and Tracking Configuration

Figure 19 - IoU, distance, type, dimensions, and detector limits

## 9.1 Select a detector

| Detector | Use when | Parameters to fix |
| --- | --- | --- |
| Find Blobs | Background and objects have stable colour contrast | Colour filter, min/max width/length |
| Find Circles | Parts are circular with clear edges | Radius and centre distance |
| External Script | Several classes or complex shapes require AI/YOLO | Model, class-to-type map, coordinateSpace |

## 9.2 Tune association

- DistanceThreshold must exceed v_belt x frame_period + 2 x detection_error but remain below minimum part separation.

- IoUThreshold must tolerate inter-frame movement without allowing nearby parts to exchange UIDs.

- One detection associates with at most one track per cycle. Monitor ID, Type, X, Y, Angle, and Is Picked.

- Tracking rejects new claims when detections or encoder data exceed the stale limit (2 seconds by default).

Figure 20 - #Objects table for UID, type, pose, angle, and picked state

## 9.3 External detector

Figure 21 - Select External Script as the detecting algorithm

Figure 22 - YOLO detection with bounding box and angle

{
  "type": "objects",
  "coordinateSpace": "image",
  "listName": "#Objects",
  "list": [
    {"type": 0, "x": 412.5, "y": 238.0, "z": 0,
     "w": 35, "h": 62, "angle": 17, "isPicked": false}
  ]
}

DATA CONTRACT: The detector never assigns UIDs. Use coordinateSpace=image for pixel x/y; use conveyor only when the detector returns calibrated C coordinates in millimetres.

# 10. Multi-Robot G-Script Configuration

Use exactly one vision loop for tracking0 and one worker loop per robot. Never run multiple vision threads that publish into the same tracking ID.

| Thread | Script | Responsibility |
| --- | --- | --- |
| Vision | 00-vision-tracking.gcode | Continuously capture, detect, and commit correlated frames |
| Robot 0 | 10-robot0-type0.gcode | Claim type 0 in robot0 zone, pick, and place |
| Robot 1 | 11-robot1-type1.gcode | Claim type 1 in robot1 zone, pick, and place |

## 10.1 Vision loop

Figure 23 - Load multi-robot-sorting and open the vision loop in the G-Script editor

; One and only one vision loop for tracking0.
N25 LABEL VISION_LOOP
N30 M98 PcaptureAndDetect(0)
N50 M98 Pdelay(20)
N55 JUMP VISION_LOOP

CURRENT EXAMPLE: The current vision example uses PcaptureAndDetect, which waits for FrameReady. Do not add a fixed detector delay; use the primitive timeout and measured telemetry.

## 10.2 Worker loop and atomic claim

Figure 24 - robot0 worker: claim, Map, G01 SYNC, complete, and place

M98 PclaimObject(0,#R0Target,robot0,-180,180,300,450,0,30000)

IF #R0Target.Found == 1
    #R0Pick = #0.ConveyorToRobot0.Map(#R0Target.X,#R0Target.Y)
    G01 X[#R0Pick.X] Y[#R0Pick.Y] Z[#R0SafeZ] F1200 SYNC
    G01 X[#R0Pick.X] Y[#R0Pick.Y] Z[#R0PickZ] F500 SYNC
    M03
    M98 Pdelay(60)
    M98 PcompleteObject(0,#R0Target.UID,robot0)
ENDIF

| Primitive | When to use | Rule |
| --- | --- | --- |
| claimObject | Before a robot approaches a part | Continue only when Found=1 |
| releaseObject | Abandoning a part before pickup | Use the matching UID and owner |
| completeObject | After the end effector secures the part | Never complete before pickup confirmation |

OWNERSHIP: On ReleaseRejected or CompleteRejected, stop the worker in a controlled manner. Never continue with an old pose when ownership is uncertain.

# 11. Operating Sequence

## 11.1 Start of shift

- Inspect robot workspaces, E-stop, guards, air/vacuum, and obstructions while the conveyor is stopped.

- Open the software, load the approved project/recipe, and confirm G-Script paths.

- Connect robots, conveyor, encoder, and camera. Confirm that every COM port and IP address maps to the intended device.

- Home each robot at low speed; verify SafeZ and a safe end-effector state.

- Reset the encoder at the reference and verify sign and stable velocity at low belt speed.

- Run the vision thread only; confirm stable UIDs through the ROI and live updates in #Objects.

- Dry-run robot0 with the end effector disabled or PickZ raised. Perform a real pick with one robot only after the dry run passes.

- Enable additional robots one at a time while monitoring claim owners and non-overlapping workspaces.

RUN CONDITIONS: Camera/detector and encoder are fresh; mapping has passed validation; robots are homed; the vision loop is active; and no stale track or claim remains from a previous run.

## 11.2 Production monitoring

| Observation | Normal | Stop condition |
| --- | --- | --- |
| UID | Stable throughout the ROI | UID exchange or ghost track |
| Encoder | Continuous position and velocity | Frozen, sign jumps, or stale |
| Claim | One owner per UID | Two workers on one part or repeated lease expiry |
| Pick | Stable error | Error grows with position or speed |
| Cycle time | Within the approved baseline | Increase caused by inference, wait, or robot timeout |

## 11.3 Normal stop

- Stop feeding new parts and let claimed parts complete or release.

- Stop robot workers, then stop the vision loop.

- Stop the conveyor, move robots to approved safe poses, and disable end effectors.

- Save only approved project/configuration changes; never persist unreviewed test values.

# 12. Fault Recovery and Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| No part can be claimed | Stale detector/encoder or wrong zone/type | Check freshness, C.X/C.Y, and typeFilter |
| UID changes between frames | Distance/IoU too strict or latency too high | Measure motion per frame and retune thresholds |
| Pick error grows with Y | Incorrect mm/count or BeltVector | Recalibrate encoder and SYNC vector |
| Pick error at ROI edges | Insufficient warp or P-to-C mapping | Correct perspective or use Cloud Point Mapping |
| Position correct, angle wrong | AngleOffset, sign, or mirror | Test 0/45/90 deg; correct sign before offset |
| CompleteRejected | Expired lease, wrong owner, or changed UID | Stop the worker and discard the old target |
| GigE camera not listed | Missing vendor runtime, NIC, or subnet | Install pylon/MVS and verify IP/firewall policy |
| G-Script help missing | GScript_Documentation.html absent | Restore the file in the portable application directory |

CAMERA OR ENCODER LOSS: Stop part feeding and robots through the cell state machine. Shared tracking blocks new claims when stale but does not replace cell-level controlled-stop logic.

## Recover a worker terminated after claiming

- Stop the conveyor and robot safely, then determine whether the part was physically picked.

- If not picked, wait for lease expiry or release with the matching owner/UID. Never delete a track while its worker still runs.

- If picked but not completed, handle the part through the approved recovery procedure and clear state under supervisor authority.

- Run vision only and verify a stable shared map before restarting any worker.

# 13. Cell Acceptance

| Area | Test | Acceptance criterion | Result |
| --- | --- | --- | --- |
| Encoder | Five 1000 mm cycles | Error within pick budget | ___ |
| P-to-C mapping | Nine points across ROI | Maximum/RMS error accepted | ___ |
| C-to-Ri mapping | Three points per robot | Within suction/grip radius | ___ |
| Tracking | 100 consecutive parts | No out-of-policy UID exchange/ghost | ___ |
| Ownership | Two workers compete for one part | Exactly one Found=1 | ___ |
| Lease | Terminate worker after claim | Part unlocks only after lease | ___ |
| Stale input | Disconnect encoder/camera | No new claim; cell stops | ___ |
| Throughput | Run target speed | Cycle time and reject rate accepted | ___ |
| Safety | Test E-stop and guard | Stops independently of PC/G-Script | ___ |

PRODUCTION GATE: Enter production only after every item passes and the acceptance record is approved. Collision avoidance, pickup sensors, recipe/bin logic, and the cell fault state machine require separate assessment.

## Calibration record

| Parameter | Value | Date / performed by |
| --- | --- | --- |
| Camera resolution / exposure / ROI | ____________________________ | ________________ |
| mm_per_count / encoder sign | ____________________________ | ________________ |
| Mapping P->C max/RMS | ____________________________ | ________________ |
| ConveyorToRobot0 max error | ____________________________ | ________________ |
| ConveyorToRobot1 max error | ____________________________ | ________________ |
| DistanceThreshold / IoU | ____________________________ | ________________ |
| Detector latency / frame period | ____________________________ | ________________ |
| LeaseMs / stale timeout | ____________________________ | ________________ |

# 14. Parameter and Path Reference

| Asset | Repository path |
| --- | --- |
| Design guide | docs/multi-robot-conveyor-sorting.md |
| Vision loop | script-example/multi-robot-sorting/00-vision-tracking.gcode |
| Robot0 worker | script-example/multi-robot-sorting/10-robot0-type0.gcode |
| Robot1 worker | script-example/multi-robot-sorting/11-robot1-type1.gcode |
| Test ownership | tests/tracking_claim/ |

## Claim result variables

#result.Found
#result.UID
#result.Type
#result.X  #result.Y  #result.Z
#result.W  #result.L  #result.A
#result.ClaimOwner
#result.ClaimExpiresAt

CURRENT BOUNDARY: The platform provides shared tracking, atomic claims, leases, type/pose, and sample workers. A production cell must additionally validate multi-robot collision prevention, pickup quality confirmation, recipe/bin logic, OEE, and the cell-level fault/recovery state machine.